# Lattice Structure EDM Project — Full Report

Phase 1 & 2 analysis, geometry (pore 235.6 um, tool 900 um), 16 lab runs, SEM validation, and final EDM recommendations.

# 1. Introduction

Goal: nearly circular machined pore with continuous supporting-material (black) boundary. Nodes may be destroyed. Tool 900 um; pore 235.6 um; unit cell 500 um; tool/pore ratio 3.82.

# 2. Geometry

Lattice geometry

| parameter | value_um | source | notes |
| --- | --- | --- | --- |
| unit_cell_side | 500 | SEM scale bar / user calculation | Width of square unit cell |
| pore_diameter | 235.6 | user calculation | 3x = 500*sqrt(2); x = 235.6 um |
| node_diameter_assumed | 235.6 | user assumption | pore diameter = node diameter = x |
| unit_cell_diagonal | 707.1 | calculated | 500 * sqrt(2) |
| tool_tip_diameter | 900 | lab document / handwritten notes | EDM electrode |
| tool_to_pore_diameter_ratio | 3.82 | calculated | 900 / 235.6 - tool much larger than pore |
| pore_radius | 117.8 | calculated | 235.6 / 2 |
| tool_radius | 450 | calculated | 900 / 2 |
| working_area_3x3_cells | 1500 | calculated | 3 unit cells for 900 um tool footprint |
| clearance_if_centered | -332.2 | calculated | tool_radius - pore_radius (overlap into nodes/struts) |

Figure 5: Pore diameter 235.6 um calculation

# 3. Problem diagrams

Figure 1: Lattice structure, 900 um tool, reference (0,0)

Figure 2: Unknown tool position — Phase 1

Figure 3: Favourable vs unfavourable outcomes

Figure 4: Target — black supporting ring circular

# 4. LLM / grid ideation

Phase 2 working area: 3x3 unit cells = 1500 x 1500 um (tool exceeds single 500 um cell). Grid points (a,b) map tool landing positions for circularity prediction.

Figure 6: Project overview — 500 um unit cell, grid logic

Figure 7: Grid subdivision and ML circularity map idea

# 5. Experimental data

Figure 8: Lattice unit cell and 16-run data table

16 experimental runs

| Run | Peak_Current_A | Pulse_On_Time_us | Duty_Factor_pct | Tool_Wear_Rate | Volume_Removal_Rate | Hole_Dev_Top_um | Hole_Dev_Bottom_um |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 50 | 56 | -83.46 | 0.56 | 231.56 | 213.91 |
| 2 | 4 | 75 | 64 | -112.87 | 0.69 | 217.78 | 199.06 |
| 3 | 4 | 100 | 72 | 408.11 | 0.51 | 230.15 | 212.08 |
| 4 | 4 | 150 | 80 | -268.75 | 0.52 | 270.55 | 213.04 |
| 5 | 6 | 50 | 64 | 75.95 | 0.47 | 205.29 | 143.39 |
| 6 | 6 | 75 | 56 | 68.64 | 0.65 | 225.36 | 172.72 |
| 7 | 6 | 100 | 80 | 154.33 | 0.54 | 280.53 | 130.03 |
| 8 | 6 | 150 | 72 | 180.31 | 0.55 | 219.68 | 53.86 |
| 9 | 8 | 50 | 72 | 91.02 | 0.40 | 240.17 | 96.38 |
| 10 | 8 | 75 | 80 | 282.03 | 0.62 | 260.48 | 122.89 |
| 11 | 8 | 100 | 56 | 96.00 | 0.97 | 240.25 | 213.69 |
| 12 | 8 | 150 | 64 | 246.99 | 1.48 | 213.86 | 216.67 |
| 13 | 10 | 50 | 80 | 489.40 | 0.62 | 238.61 | 122.43 |
| 14 | 10 | 75 | 72 | 90.69 | 0.77 | 251.62 | 155.43 |
| 15 | 10 | 100 | 64 | 418.52 | 0.84 | 227.14 | 204.55 |
| 16 | 10 | 150 | 56 | 490.57 | 0.98 | 231.89 | 26.84 |

Figure 9: SEM images runs 1-16

SEM visual labels

| Run | Boundary_circularity_1to5 | Supporting_boundary_intact | Nodes_damaged_ok | Notes |
| --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 1 | Irregular boundary; supporting material destroyed |
| 2 | 2 | 0 | 1 | Not circular; boundary damaged |
| 3 | 1 | 0 | 1 | Irregular; supporting destroyed |
| 4 | 5 | 1 | 1 | TARGET: nearly circular hole; supporting material forms circular boundary (SEM reference) |
| 5 | 2 | 0 | 1 | Lower deviation numbers but boundary not circular in SEM |
| 6 | 2 | 0 | 1 | Boundary irregular in SEM |
| 7 | 1 | 0 | 1 | Highly irregular; supporting destroyed |
| 8 | 2 | 0 | 1 | Asymmetric; boundary not circular |
| 9 | 2 | 0 | 1 | Moderate but boundary irregular |
| 10 | 1 | 0 | 1 | Elongated irregular hole; supporting destroyed |
| 11 | 1 | 0 | 1 | Irregular boundary |
| 12 | 2 | 0 | 1 | Similar top/bottom deviation but SEM shows non-circular boundary |
| 13 | 1 | 0 | 1 | Irregular; supporting destroyed |
| 14 | 2 | 0 | 1 | Boundary not circular |
| 15 | 2 | 0 | 1 | Boundary not circular |
| 16 | 1 | 0 | 1 | Highly irregular despite low bottom deviation |

# 6. Methodology

Extract 16 runs; label SEM (Run 4 = only success)

Phase 1: unknown position — GP + LOOCV on boundary circularity

Phase 2: known (x,y) in 1500 um working area

Compare with/without SEM images

Scripts: phase1_model_actual.py, phase1_model.py

# 7. Final results

Phase 1 WITH images: 4 A, 150 us, 80 % (Run 4)

Phase 1 WITHOUT images: 6 A, 50 us, 64 % — NOT recommended

Phase 2 WITH images: 4 A at center; 3.5 A near struts

Recommended trials

| Phase | Use_SEM_images | Position | Peak_Current_A | Pulse_On_Time_us | Duty_Factor_pct | Pore_diameter_um | Tool_diameter_um | Priority | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Yes | Unknown | 4 | 150 | 80 | 235.6 | 900 | PRIMARY | Run 4 - tool/pore ratio 3.82 |
| 1 | Yes | Unknown | 3.5 | 150 | 80 | 235.6 | 900 | Secondary | Softer for small pore |
| 1 | Yes | Unknown | 4 | 140 | 78 | 235.6 | 900 | Secondary | Near Run 4 |
| 1 | No | Unknown | 6 | 50 | 64 | 235.6 | 900 | Not recommended | Deviation only - breaks boundary |
| 2 | Yes | Center | 4 | 150 | 80 | 235.6 | 900 | PRIMARY | Pore center in 500um cell |
| 2 | Yes | Near strut | 3.5 | 150 | 78 | 235.6 | 900 | PRIMARY | 900um tool near 500um strut |
| 2 | Yes | Near node_0_0 | 3.5 | 145 | 76 | 235.6 | 900 | PRIMARY | Corner reference |
| 2 | Yes | Mid pore | 4 | 148 | 79 | 235.6 | 900 | Secondary | Between center and strut |
| 2 | No | Center | 6 | 100 | 64 | 235.6 | 900 | Not recommended | No SEM validation |

# 8. Conclusion

Use Run 4 parameters with fine feed and stable gap. Synthetic 1100 points are AI-generated, not independent lab data. Future: web tool for circularity heatmap over 1500x1500 um grid.